PAGE  
1

ROBUST STUDY SUMMARY - Persistence
	Item
	Yes
	No

	Reference: USEPA Robust Summary, FND Amines Category, December, 2002.  201-14171B


	Test Substance   CAS # and name (specify, but do not assess this item): 112-18-5; N,N-Dimethyl-1-dodecanamine

	Substance purity reported? (Y/N and specify) 
	
	X

	Method

	References (Y/N)
	X
	

	OECD, EU, national, or other standard method? (Y/N)
	X
	

	If not a standard method, justification of the method/protocol provided? (Y/N)
	n/a
	

	Test design / conditions 

	Study type (e.g., hydrolysis, biodegradation, etc.  – specify, but do not assess): biodegradation

	Conditions type (aerobic or anaerobic - specify, but do not assess): aerobic

	Test medium (air, water, soil, or sediment - specify, but do not assess): water

	Information on stability of the substance in the media of concern available? (do not assess this item)  No

	Information on controls (Y/N and specify, positive or negative)  positive and negative
	X
	

	Number of replicates (Y/N and specify) 2 
	X
	

	Temperature (Y/N and specify)  19.7-24 degrees C
	X
	

	Test duration (Y/N and specify) 29days
	X
	

	Analytical method / technique used (Y/N)
	
	X

	For photodegradation only

	Reactants of gas-phase reactions (specify, but do not assess this item)

	Light source (Y/N and specify)
	
	

	Light spectrum and/or relative intensity based on sunlight intensity (Y/N)
	
	

	For hydrolysis only

	Measured concentrations reported? (Y/N)
	
	

	pH values reported? (Y/N and specify)  
	
	

	For biodegradation only

	Ready or inherent biodegradation? (specify, but do not assess this item) Ready

	Substance concentration (Y/N)
	X
	

	Inoculum source (Y/N)
	X
	

	Inoculum concentration or number of microorganisms (Y/N)
	X
	

	Results

	Endpoints / values / units (do not assess this item):  72% after 29days

	Information on breakdown products available? (do not assess this item) No

	Overall score:  9/11=82%

	EC Reliability  code: 1

	Reliability category (high, satisfactory, low):  High

	Comments:  OECD Procedure 301B and Procedure C,4-C of the Annex to Directive 92/69/EEC.



